# SheShield Companion

SheShield Companion is a connected personal-safety system that combines an ESP32 wearable, a React Native mobile app, GPS location sharing, emergency SMS, and a consent-based live camera session for trusted assistance.

The project is designed to help a user send an emergency alert quickly while sharing useful context with an authorized contact. The system supports Bluetooth Low Energy telemetry from a wearable, phone-based location capture, a configured SMS alert, and an authenticated web viewer for a temporary LiveKit session.

Safety notice: SheShield is a prototype and should not be treated as a replacement for local emergency services. Test the hardware, phone permissions, network coverage, SMS delivery, and trusted-viewer access before relying on it.

## Main Features

- Manual SOS: The user can trigger an emergency event from the mobile app or connected wearable.

- Emergency SMS: The configured alert flow sends an SOS message to 8825542451 with the freshest available GPS coordinates and a Google Maps link.

- Wearable telemetry: The app connects to an ESP32 wearable over BLE using a UART-style service.

- Sensor support: The reference hardware supports motion, pulse/PPG contact quality, GPS, battery, and GSM fallback components.

- Live location: The app requests location permission and attaches coordinates to the emergency alert when available.

- Consented live camera: After an SOS event, the user must explicitly approve camera access before a LiveKit video session starts.

- Authenticated viewer: A trusted viewer signs in and receives a short-lived subscriber token for the active SOS room.

- Visible streaming state: The app shows when camera sharing is active and provides a stop control.

- Event history: Emergency and wearable events can be reviewed in the app.

- Contacts and settings: The app includes screens for contacts, connection preferences, permissions, and safety settings.

- GSM fallback: The hardware documentation includes SIM800L-based SMS fallback for situations where the phone or internet path is unavailable.

## System Overview

ESP32 wearable

├── Motion sensor

├── Pulse/PPG sensor

├── GPS module

├── Battery status

└── BLE UART / GSM fallback

│

▼

SheShield mobile app

├── Wearable connection

├── SOS control

├── GPS capture

├── Emergency SMS flow

├── Event history

└── Explicit camera consent

│

├── Short-lived LiveKit publisher session

▼

SheShield backend

├── Authenticated API routes

├── Event persistence

├── LiveKit token generation

└── Trusted viewer authorization

│

▼

Authenticated web viewer

└── Watches the active SOS video session

## Repository Structure

app/             Expo Router screens and mobile routes

components/      Dashboard, maps, SOS camera, and reusable UI

constants/       App constants and theme configuration

docs/            Hardware, BLE, LiveKit, and project documentation

drizzle/         Database schema and migrations/metadata

firmware/        ESP32 sketches and hardware reference code

hooks/           React hooks for authentication and app state

lib/             BLE, SMS, GPS, theme, API, and runtime helpers

server/          Express/tRPC API, authentication, persistence, and LiveKit tokens

shared/          Shared types and constants

scripts/         Development and utility scripts

## Technology Stack

### Mobile

- Expo SDK 57

- React Native

- Expo Router

- NativeWind

- Expo Camera

- Expo Location

- Expo SMS

- Expo Notifications

- React Native BLE PLX

- React Native Maps

- LiveKit React Native and WebRTC bindings

### Backend

- Node.js

- Express

- tRPC

- Drizzle ORM

- MySQL-compatible managed database

- LiveKit Server SDK for short-lived room tokens

### Hardware

- ESP32 development board

- MPU6050 motion sensor

- MAX30102 pulse/PPG sensor

- NEO-6M GPS module

- SIM800L GSM module

- Rechargeable battery and charging circuit

See docs/SheShield_Hardware_Connections.md for wiring and pin details.

## Local Development

### Requirements

- Node.js 20 or newer

- pnpm

- Expo-compatible Android or iOS development environment

- ESP32 hardware for BLE testing

- A configured database for backend development

- A LiveKit Cloud project for live camera testing

### Install Dependencies

pnpm install

### Configure Environment Variables

Create a local environment file from the example template when available. Server-only LiveKit values must never be placed in the mobile bundle or committed to Git:

LIVEKIT_URL=wss://your-project.livekit.cloud

LIVEKIT_API_KEY=your_server_api_key

LIVEKIT_API_SECRET=your_server_api_secret

The mobile app may use a browser-safe API base URL, but it must not contain the LiveKit API secret.

### Start the Development Project

pnpm dev

The managed project uses:

- Metro/Web preview: http://localhost:8081

- Backend API: http://localhost:3000

Useful commands:

pnpm check       # TypeScript validation

pnpm lint        # Expo lint checks

pnpm db:push     # Apply the Drizzle schema

pnpm test        # Run the configured test suite

## SOS Flow

- The user presses SOS in the app or triggers the wearable emergency input.

- The app records the event and requests the freshest available location.

- The configured SMS flow sends the alert to 8825542451 with a Google Maps location link.

- The app shows a clear camera-consent prompt.

- If the user approves, the app requests a short-lived LiveKit publisher token.

- The mobile app publishes the camera stream to the SOS room.

- An authenticated trusted viewer requests a short-lived viewer token and joins the room.

- The user can stop camera sharing at any time, and the session is disconnected when the SOS flow ends.

## Privacy and Security

- Camera access is never started silently; explicit user consent is required.

- A visible in-app status indicates when live camera sharing is active.

- LiveKit API credentials remain on the server and are not bundled into Expo output.

- Publisher and viewer tokens are short-lived and scoped to the SOS room.

- Viewer access requires authentication.

- GPS data should be treated as sensitive personal information.

- Do not commit .env, API secrets, private keys, or production contact lists.

- Use test phone numbers and test rooms during development.

- Review local privacy, consent, emergency-contact, and data-retention requirements before deployment.

## Hardware Bring-Up

- Assemble the ESP32 and sensors according to the wiring guide.

- Flash the selected firmware sketch from firmware/.

- Confirm power and ground connections before attaching sensors.

- Test the BLE UART advertisement and pairing.

- Verify motion, PPG contact, GPS, and battery packets individually.

- Test the manual SOS input without sending alerts to a real emergency contact.

- Test the SIM800L path with a controlled test number.

- Connect the mobile app and confirm event history updates.

The firmware sketches are reference prototypes. Confirm voltage levels, current requirements, antenna placement, and enclosure safety before field use.

## Testing Checklist

- The mobile app starts without missing native permissions.

- The wearable is discovered over BLE.

- Telemetry packets are displayed in the app.

- GPS permission and location refresh work on the target device.

- The SOS message contains the expected maps link.

- The test SMS reaches the configured test contact.

- Camera permission is requested only after SOS consent.

- The LiveKit publisher connects with a short-lived token.

- An authenticated viewer can join the active room.

- An unauthenticated viewer is rejected.

- The Stop live camera action disconnects the room.

- The GSM fallback is tested separately from the internet path.

- Battery, reconnect, and network-loss behavior are documented.

## Documentation

- Hardware connections

- BLE UART and fallback

- LiveKit SOS viewer

- Complete project document

- Companion app and Bluetooth notes

## Roadmap

- Complete native Android and iOS development builds.

- Validate the wearable enclosure, power budget, and field connectivity.

- Add robust reconnection handling for BLE and GSM paths.

- Add contact verification and safer production contact management.

- Perform controlled testing with documented false-alert and missed-alert cases.

- Review privacy, consent, data retention, and emergency-response procedures with project mentors.

- Prepare a limited pilot only after hardware, network, and safety validation.

## License

Add the project license selected by the team before publishing this repository. If the repository contains third-party libraries, firmware, images, or documentation, retain their original license and attribution notices.